Saad Aljammaz

435105061

Pure Aloha

The node immediately transmits its frame completely

If the frame is collided it retransmits the frame again with the probability p.

Slotted Aloha

Frames are of the same size

time is divided into equal size slots

Operation

- when node obtains fresh frame, it transmits in next slot

- no collision, node can send new frame in next slot

- if collision, node retransmits frame in each subsequent slot with prob. p until success

# Lecture 2

Carrier Sense Multiple Access(CSMA)

# Invented to minimize collisions and increase the performance

A node should not send if another node is already sending

# Persistence methods:

1. Non-persistent strategy

2. Persistent strategy

3. P-persistent strategy

CSMA with Collision Detection (CSMA/CD)

There are many collision detection methods!

- detecting voltage level on the line

- detecting power level

- detecting simultaneous transmission & reception

CSMA with Collision Avoidnes (CSMA/CA)

Priorities:

SIFS : highest priority, forACK, CTS, pollingresponse

PIFS : medium priority, for time-bounded service using PCF

DIFS : lowest priority, for asynchronous data service

Random Contention Access

# Slotted contention period:

- Used by all carrier sense variants

- Provides random access to the channel

Contention Window

- Random number selected from [0,cw].

- Small value for cw

- Optimal cw for known number of contenders & know packet size

# 802.11 - CSMA/CA unicast

Sending unicast packets :

- Station has to wait for DIFS before sending data

- Receiver acknowledges at once (after waiting for SIFS ) if the packet was received correctly (CRC)

- Automatic retransmission of data packets incase of transmission errors

Procedure :

- Similar to CSMA but instead of sending packets control frames are exchanged

- RTS = request to send

- CTS = clear to send

- DATA = actual packet

- ACK = acknowledgement

Advantages :

- Small control frames lessen the cost of collisions (when data is large)

- RTS + CTS provide “virtual” carrier sense which protects against hidden terminal collisions (where A can’t hear B)

# 802.11 DCF (CSMA-CA)

- Full exchange with “virtual” carrier sense (called the Network Allocation Vector)

# Carrier Sense Multiple Access (CSMA)

Procedure

- Listen to medium and wait until it is free (no one

- else is talking)

- Wait a random back off time then start talking

Advantages

- Fairly simple to implement

- Functional scheme that works

Disadvantages

- Can not recover from a collision
(inefficient waste of medium time)

# Virtual Carrier Sense

- Provided by RTS & CTS

- Designed to protect against hidden terminal collisions (when C can’t receive from A and might start transmitting)

# Lecture 2
Controlled Access Protocols

It means which station has the right to send

# Reservation

# Polling

# Token passing

# Lecture 3

Channelization Protocols

It is multiple-access method in which the available bandwidth of a link is shared in Time , frequency or through code.

# Frequency-division multiple access (FDMA)

In FDMA, the available bandwidth of the common channel is divided into bands that are separated by guard bands.

# Time-division multiple access (TDMA)

the bandwidth is just one channel that is timeshared between different stations.

Code-Division Multiple Access (CDMA)

In CDMA, one channel carries all transmissions simultaneously.

# Lecture 3

Local Area Network
(Ethernet)

It is the dominant LAN technology.

- Cheap

- First widely used LAN technology

- Simpler and cheaper than token LANs

- Kept up with speed race: 10, 100, 1000 Mbps

Physical layer

# Physical layer is dependent on the implementation and type of the physical media used.

- IEEE define detailed specifications for each LAN implementation.

Ethernet evolution

# Ethernet Frame Format

- Sending adapter encapsulates network layer protocol packet such as IP datagram in Ethernet frame

# Preamble:

- 7 bytes with pattern 10101010.

- Used to synchronize receiver, sender clock rates.

# SFD:

- One byte with pattern 10101011 to signal the start of the frame.

# Addresses:

6 bytes, frame is received by all adapters on a LAN and dropped if address does

not match

# Length/Type:

indicates the higher layer protocol or the number of bytes in the data field.

# CRC:

checked at receiver, if error is detected, the frame is simply dropped

# Minimum and Maximum Lengths

Minimum: 64 bytes (512 bits)

Maximum: 1518 bytes (12,144 bits)

# Unicast and multicast addresses

The least significant bit of the first byte defines the type of address. If the bit is 0, the address is unicast; otherwise, it is multicast or broadcast.

The broadcast destination address is a special case of the multicast address in which all bits are 1s. (FF:FF:FF:FF:FF:FF)

# Slot time

# Slot time = round-trip time+ jam sequence time.

Max Length = propagation speed * (slot time/2)

# WIRELESS LANS

# Basic service sets (BSSs)

# Extended service sets (ESSs)

# 802.11 Wireless LAN

A client is always associated with one AP and when the client moves closer to another AP, it associates with the new AP (Hand-Off)

# Wireless LAN Protocols

Wireless has complications compared to wired.

Nodes may have different coverage regions

- Leads to hidden and exposed terminals

Nodes can’t detect collisions, i.e., sense while sending

- Makes collisions expensive and to be avoided

# Wireless LANs – Hidden terminals

Hidden terminals are senders that cannot sense each other but nonetheless collide at intended receiver

- Want to prevent; loss of efficiency

- S1 and S2 are hidden terminals when sending to R

# Hidden Terminal problem

Before every data transmission

- Sender sends a Request to Send (RTS) frame containing the length of the transmission

- Receiver respond with a Clear to Send (CTS) frame

- Sender sends data

- Receiver sends an ACK; now another sender can send data

When sender doesn’t get a CTS back, it assumes collision

# Exposed Terminal problem

Exposed terminals are senders who can sense each other but still transmit safely (to different receivers)

- Desirably concurrency; improves performance

- S1 > R1 and S2 > R2 are exposed terminals

# Wireless LANs – MACA

MACA protocol grants access for A to send to B:

- A sends RTS to B;B replies with CTS.

- A can send with exposed but no hidden terminals